TCP/IP & Protocols

1. What is TCP/IP?

TCP/IP is the actual protocol suite the internet runs on. While OSI is a theoretical model, TCP/IP is what's really being used.

It has 4 layers (maps to OSI but simpler):

| TCP/IP Layer | Maps to OSI Layers |
| --- | --- |
| Application | 7, 6, 5 |
| Transport | 4 |
| Internet | 3 |
| Network Access | 2, 1 |

Don't overthink the mapping. Just know TCP/IP is the real implementation, OSI is the reference model.

2. TCP — Transmission Control Protocol

TCP is reliable and connection-based. Before sending data, it establishes a connection using the 3-Way Handshake.

The 3-Way Handshake

Client          Server

|--- SYN ------->|   "I want to connect"

|<-- SYN-ACK ----|   "Okay, I acknowledge"

|--- ACK ------->|   "Great, connection established"

- SYN — synchronize

- ACK — acknowledge

After this, data flows. When done, connection is closed with a 4-way FIN handshake.

What TCP guarantees:

- Data arrives in order

- Data arrives completely (re-transmits lost packets)

- Error checking on every segment

Use cases:

- Web browsing (HTTP/HTTPS)

- Email

- File transfers

- Anything where accuracy matters

Why it matters for hacking: The SYN handshake is the basis of SYN flood attacks and port scanning. When Nmap scans ports, it's sending SYN packets and watching what comes back.

3. UDP — User Datagram Protocol

UDP is fast and connectionless. No handshake, no confirmation, just fire and forget.

What UDP does NOT guarantee:

- Order of delivery

- Complete delivery

- Error recovery

Use cases:

- Video streaming

- Online gaming

- VoIP (voice calls)

- DNS lookups

- Anything where speed matters more than perfection

TCP vs UDP — Simple Comparison

| Feature | TCP | UDP |
| --- | --- | --- |
| Connection | Yes (handshake) | No |
| Reliability | High | Low |
| Speed | Slower | Faster |
| Order guaranteed | Yes | No |
| Use case | Web, email, files | Streaming, gaming, DNS |

4. IP — Internet Protocol

IP handles addressing and routing — getting packets from source to destination.

Two versions:

- IPv4 — 32-bit address, e.g. 192.168.1.1

- IPv6 — 128-bit address, e.g. 2001:db8::1

IP is connectionless and unreliable on its own — it just routes packets. TCP sits on top of IP to add reliability. That's why it's called TCP/IP together.

5. ICMP — Internet Control Message Protocol

ICMP is used for network diagnostics and error reporting. It's not for transferring data.

- Ping uses ICMP — sends an echo request, waits for echo reply

- Traceroute uses ICMP — maps the hops between you and a destination

bash

ping google.com

traceroute google.com

Why it matters for hacking: Ping tells you if a host is alive. ICMP can also be used in ICMP flood attacks. Some firewalls block ICMP to hide hosts — so if ping fails, the host might still be up.

6. Key Application Layer Protocols

These are protocols you'll encounter constantly in red teaming.

HTTP — HyperText Transfer Protocol

- Port 80

- Used for web communication

- Plain text — anyone intercepting the traffic can read it

- Request methods: GET, POST, PUT, DELETE

Why it matters for hacking: Unencrypted. Man-in-the-middle attacks can intercept and read everything. Also the main surface for web app attacks.

HTTPS — HTTP Secure

- Port 443

- HTTP + TLS encryption

- Data is encrypted in transit

- Uses SSL/TLS certificates to verify identity

Why it matters for hacking: Encrypted but not unbreakable. SSL stripping, certificate spoofing, and misconfigured TLS are attack vectors.

FTP — File Transfer Protocol

- Port 21 (control), Port 20 (data)

- Used to transfer files between client and server

- Plain text — credentials sent unencrypted

- Anonymous FTP login is a common misconfiguration

Why it matters for hacking: Frequently misconfigured, credentials sniffable, anonymous login sometimes enabled.

SSH — Secure Shell

- Port 22

- Encrypted remote login to a machine

- Replaced Telnet (which was plain text)

- Uses key-based or password authentication

Why it matters for hacking: Finding SSH open on a target is valuable. Brute forcing SSH passwords, weak keys, and misconfigurations are common attack paths.

Telnet

- Port 23

- Old remote login protocol

- Completely plain text — everything including passwords visible

- Almost never should be running in 2025

Why it matters for hacking: If you find Telnet open, it's a serious vulnerability. Easy to intercept credentials.

SMTP — Simple Mail Transfer Protocol

- Port 25 (or 587 for modern use)

- Used to send emails

- Can be abused for spam, phishing infrastructure

DNS — Domain Name System

- Port 53

- UDP for normal lookups, TCP for large responses

- Translates domain names to IPs

Why it matters for hacking: DNS enumeration reveals subdomains and infrastructure. DNS zone transfers (misconfigured servers) can dump an entire domain's records. DNS can also be used for data exfiltration.

DHCP — Dynamic Host Configuration Protocol

- Port 67/68

- Automatically assigns IP addresses to devices on a network

- Your router runs DHCP — that's how your laptop gets an IP automatically

Why it matters for hacking: DHCP spoofing — attacker sets up rogue DHCP server, assigns themselves as the gateway, intercepts all traffic.

ARP — Address Resolution Protocol

- No port (operates at Layer 2)

- Translates IP addresses to MAC addresses within a local network

- "Who has IP 192.168.1.1? Tell me your MAC address."

Why it matters for hacking: ARP spoofing is one of the most common LAN attacks. You broadcast fake ARP replies to poison devices' ARP tables, redirecting their traffic through you. Foundation of man-in-the-middle attacks on local networks.

7. How It All Works Together — Full Picture

When you type https://google.com and hit enter:

- DNS lookup — resolves google.com to an IP (Layer 7, UDP port 53)

- TCP 3-way handshake — establishes connection to Google's server on port 443 (Layer 4)

- TLS handshake — encrypts the connection (Layer 6)

- HTTP GET request — your browser requests the page (Layer 7)

- IP routing — packets hop through routers to reach Google (Layer 3)

- Google responds — sends back the HTML, CSS, JS

- TCP confirms delivery — all packets received, connection maintained

- Browser renders the page

Every single step involves a protocol you just learned.